第一问代码

import matplotlib.pyplot as plt

import numpy as np

from math import cos, sin, pi, acos, atan, sqrt

import seaborn as sns

sns.set()

# Constants and parameters

POP_SIZE = 200

NUM_PARAMS = 3

NUM_GENERATIONS = 100

MUTATION_RATE = 0.1

CROSSOVER_PROB = 0.8

# Initialize population

def initialize_population(pop_size, num_params):

return np.random.rand(pop_size, num_params)

# Compute end-effector position

def endpoint(x):

a1 = (320*x[0] - 160) * pi / 180

a2 = (165*x[1] - 60) * pi / 180

a3 = (280*x[2] - 200) * pi / 180

l1 = 300

R2_1 = np.array([[cos(a1), 0, sin(a1), l1*cos(a1)],

[sin(a1), 0, -cos(a1), l1*sin(a1)],

[0, 1, 0, 600],

[0, 0, 0, 1]])

l2 = 1200

R3_2 = np.array([[cos(a2), -sin(a2), 0, l2*cos(a2 + pi/2)],

[sin(a2), cos(a2), 0, l2*sin(a2 + pi/2)],

[0, 0, 1, 0],

[0, 0, 0, 1]])

l3_4 = 1500

X = np.array([l3_4*cos(pi/2 + a3), l3_4*sin(pi/2 + a3), 0, 1])

R = R2_1 @ R3_2

y = np.dot(R, X)

return y

# Fitness function (RMSE)

def fitness(x):

P = endpoint(x)

target = np.array([1500, 1200, 200])

return np.sqrt(np.sum((P[:3] - target)**2))

# Tournament selection

def tournament_selection(population, fitness_scores, tournament_size=3):

selected_parents = []

pop_size = len(population)

for _ in range(pop_size):

tournament_indices = np.random.choice(pop_size, size=tournament_size, replace=False)

tournament_fitness = fitness_scores[tournament_indices]

selected_parents.append(population[tournament_indices[np.argmin(tournament_fitness)]])

return np.array(selected_parents)

# Single-point crossover

def crossover(parents, crossover_prob=CROSSOVER_PROB):

if np.random.rand() < crossover_prob:

crossover_point = np.random.randint(1, len(parents[0]))

child1 = np.concatenate([parents[0][:crossover_point], parents[1][crossover_point:]])

child2 = np.concatenate([parents[1][:crossover_point], parents[0][crossover_point:]])

else:

child1, child2 = parents[0].copy(), parents[1].copy()

return child1, child2

# Mutation

def mutate(child, mutation_rate=MUTATION_RATE):

for i in range(len(child)):

if np.random.rand() < mutation_rate:

child[i] = np.random.rand()

return child

# Genetic algorithm

def genetic_algorithm(num_generations, pop_size, num_params):

population = initialize_population(pop_size, num_params)

best_fitness_history = []

for generation in range(num_generations):

fitness_scores = np.array([fitness(x) for x in population])

best_fitness = np.min(fitness_scores)

best_individual = population[np.argmin(fitness_scores)]

best_fitness_history.append(best_fitness)

next_population = [best_individual]  # Elitism: preserve the best individual

while len(next_population) < pop_size:

parents = tournament_selection(population, fitness_scores)

child1, child2 = crossover(parents)

child1 = mutate(child1)

child2 = mutate(child2)

next_population.extend([child1, child2])

population = np.array(next_population[:pop_size])

# Final best individual and fitness

final_fitness_scores = np.array([fitness(x) for x in population])

best_individual = population[np.argmin(final_fitness_scores)]

best_fitness = np.min(final_fitness_scores)

# 创建图像

plt.figure(figsize=(8, 6))

# 使用Seaborn的线性回归图绘制数据

sns.lineplot(x=range(1, len(best_fitness_history) + 1), y=best_fitness_history, marker='o', color='b')

# 设置标签和标题

plt.xlabel('Generation')

plt.ylabel('Fitness')

plt.title('Fitness Change in Genetic Algorithm')

# 显示网格

plt.grid(True)

# 调整布局

plt.tight_layout()

# 显示图像

plt.show()

return best_individual, best_fitness, best_fitness_history

return best_individual, best_fitness, best_fitness_history

# Run the genetic algorithm

best_individual, best_fitness, best_fitness_history = genetic_algorithm(NUM_GENERATIONS, POP_SIZE, NUM_PARAMS)

print("最优调控参数:", best_individual)

print("最小欧氏距离:", best_fitness)

'''求解后三个调控参数'''

#输出目标点在坐标系四中的坐标,为了求解关节4,5,6转角

def spacechange(x):

a1=(320*x[0]-160)*pi/180

a2=(165*x[1]-60)*pi/180

a3=(280*x[2]-200)*pi/180

l1=300

R2_1=np.array([[cos(a1),0,sin(a1),l1*cos(a1)],

[sin(a1),0,-cos(a1),l1*sin(a1)],

[0,1,0,600],

[0,0,0,1]])

l2=1200

R3_2=np.array([[cos(a2),-sin(a2),0,l2*cos(a2+pi/2)],

[sin(a2),cos(a2),0,l2*sin(a2+pi/2)],

[0,0,1,0],

[0,0,0,1]])

l3_4=1500

R4_3=np.array([[0,cos(a3),cos(pi/2+a3),l3_4*cos(pi/2+a3)],

[0,sin(a3),sin(pi/2+a3),sin(pi/2+a3)*l3_4],

[1,0,0,0],

[0,0,0,1]])

X=np.array([1500,1200,200,1])

R=R2_1@R3_2@R4_3

# 计算A的逆矩阵

R_inv = np.linalg.inv(R)

y=np.dot(R_inv, X)

return y

#输出后三位参数优化调控方向,输入前三个参数角度值

def lastthere(x):

P=spacechange(x)

x0=P[0]

y0=P[1]

z0=P[2]

cos=x0/sqrt(x0**2+y0**2)

a=acos(cos)

tan=z0/sqrt(x0**2+y0**2)

b=atan(tan)

a4=a-2*pi

a5=0

a6=pi/2-b

l=[a4,a5,a6]

return l

print('后三个角精细朝向调控参数为：',lastthere(best_individual))

第二问代码

'''层次分析AHP'''

import numpy as np

from numpy import linalg

# np.set_printoptions(precision=4)

'''算术平均法权重'''

def arithmetic_mean(a):

n = len(a)

b = sum(a)

#print('b:', b)

# 归一化处理

normal_a = a/b

#print("算术平均法权重-归一化处理：")

#print(normal_a)

average_weight = []

for i in range(n):

s = sum(normal_a[i])

#print("第{}行求和 ".format(i+1), s)

# 平均权重

average_weight.append(s/n)

# print(average_weight)

print('算术平均法权重:',np.array(average_weight))

return np.array(average_weight)

'''几何平均法求权重'''

def geometric_mean(a):

n = len(a)

# 1表示按照行相乘，得到一个新的列向量

b = np.prod(a, 1)

#print(b)

c = pow(b, 1/n)

#print(c)

# 归一化处理

average_weight = c/sum(c)

print('几何平均法权重:',average_weight)

return average_weight

'''特征值法求权重'''

def eigenvalue(a):

w, v = np.linalg.eig(a)

#for i in range(len(w)):

#print('特征值', a[i], '特征向量', v[:, i])

index = np.argmax(w)

w_max = np.real(w[index])

vector = v[:, index]

vector_final = np.transpose(np.real(vector))

print('最大特征值:', w_max, 'CR:', (w_max-3)/2)

normalized_weight = vector_final/sum(vector_final)

print('特征值法归一化处理后:', normalized_weight)

return w_max, normalized_weight

'''综合平均权重'''

def average_Weight(a):

am = arithmetic_mean(a)

gm = geometric_mean(a)

ev = eigenvalue(a)[1]

aw = np.array([am, gm, ev])

#print(aw)

final_weight = sum(aw)/3

#print(final_weight)

return final_weight

a=np.array([[1,5,7],

[1/5,1,5],

[1/7,1/5,1]])

print('综合三者权重为',average_Weight(a))

'''第二问优化'''

import matplotlib.pyplot as plt

import numpy as np

from math import cos, sin, pi, acos, atan, sqrt

# Constants and parameters

POP_SIZE = 200

NUM_PARAMS = 3

NUM_GENERATIONS = 100

MUTATION_RATE = 0.1

CROSSOVER_PROB = 0.8

# Initialize population

def initialize_population(pop_size, num_params):

return np.random.rand(pop_size, num_params)

# Compute end-effector position

def endpoint(x):

a1 = (320*x[0] - 160) * pi / 180

a2 = (165*x[1] - 60) * pi / 180

a3 = (280*x[2] - 200) * pi / 180

l1 = 300

R2_1 = np.array([[cos(a1), 0, sin(a1), l1*cos(a1)],

[sin(a1), 0, -cos(a1), l1*sin(a1)],

[0, 1, 0, 600],

[0, 0, 0, 1]])

l2 = 1200

R3_2 = np.array([[cos(a2), -sin(a2), 0, l2*cos(a2 + pi/2)],

[sin(a2), cos(a2), 0, l2*sin(a2 + pi/2)],

[0, 0, 1, 0],

[0, 0, 0, 1]])

l3_4 = 1500

X = np.array([l3_4*cos(pi/2 + a3), l3_4*sin(pi/2 + a3), 0, 1])

R = R2_1 @ R3_2

y = np.dot(R, X)

return y

#定义重力势能

def delta_Ep(x):

a1 = (320*x[0] - 160) * pi / 180

a2 = (165*x[1] - 60) * pi / 180

a3 = (280*x[2] - 200) * pi / 180

l2=1200

l1=300

R2_1 = np.array([[cos(a1), 0, sin(a1), l1*cos(a1)],

[sin(a1), 0, -cos(a1), l1*sin(a1)],

[0, 1, 0, 600],

[0, 0, 0, 1]])

P3_2=np.array([l2*cos(pi/2+a2),l2*sin(pi/2+a2),0,1])

P3=np.dot(R2_1,P3_2)

z3=P3[2]*10**(-3)

z4=endpoint(x)[2]*10**(-3)

z2=0.6

L=0.6+0.3+1.2+1.5

M=5

g=9.8

Ef=(0.3/L)*z2*M*g+(1.2/L)*M*g*(z2+z3)/2+(1.5/L)*M*g*(z4+z3)/2

E0=M*g*L/2

return E0-Ef

# Fitness function (RMSE)

def RMSE(x):

P = endpoint(x)

target = np.array([1500, 1200, 200])

return np.sqrt(np.sum((P[:3] - target)**2))

def fitness(x):

return 0.7086*RMSE(x)+0.2228*delta_Ep(x)

# Tournament selection

def tournament_selection(population, fitness_scores, tournament_size=3):

selected_parents = []

pop_size = len(population)

for _ in range(pop_size):

tournament_indices = np.random.choice(pop_size, size=tournament_size, replace=False)

tournament_fitness = fitness_scores[tournament_indices]

selected_parents.append(population[tournament_indices[np.argmin(tournament_fitness)]])

return np.array(selected_parents)

# Single-point crossover

def crossover(parents, crossover_prob=CROSSOVER_PROB):

if np.random.rand() < crossover_prob:

crossover_point = np.random.randint(1, len(parents[0]))

child1 = np.concatenate([parents[0][:crossover_point], parents[1][crossover_point:]])

child2 = np.concatenate([parents[1][:crossover_point], parents[0][crossover_point:]])

else:

child1, child2 = parents[0].copy(), parents[1].copy()

return child1, child2

# Mutation

def mutate(child, mutation_rate=MUTATION_RATE):

for i in range(len(child)):

if np.random.rand() < mutation_rate:

child[i] = np.random.rand()

return child

# Genetic algorithm

def genetic_algorithm(num_generations, pop_size, num_params):

population = initialize_population(pop_size, num_params)

best_fitness_history = []

for generation in range(num_generations):

fitness_scores = np.array([fitness(x) for x in population])

best_fitness = np.min(fitness_scores)

best_individual = population[np.argmin(fitness_scores)]

best_fitness_history.append(best_fitness)

next_population = [best_individual]  # Elitism: preserve the best individual

while len(next_population) < pop_size:

parents = tournament_selection(population, fitness_scores)

child1, child2 = crossover(parents)

child1 = mutate(child1)

child2 = mutate(child2)

next_population.extend([child1, child2])

population = np.array(next_population[:pop_size])

# Final best individual and fitness

final_fitness_scores = np.array([fitness(x) for x in population])

best_individual = population[np.argmin(final_fitness_scores)]

best_fitness = np.min(final_fitness_scores)

# 绘制适应度变化曲线

plt.figure(figsize=(8, 6))

plt.plot(range(1,len(best_fitness_history)+1), best_fitness_history, marker='o', linestyle='-', color='b')

plt.xlabel('Generation')

plt.ylabel('Fitness')

plt.title('Fitness Change in Genetic Algorithm')

plt.grid(True)

plt.tight_layout()

plt.show()

return best_individual, best_fitness, best_fitness_history

# Run the genetic algorithm

best_individual, best_fitness, best_fitness_history = genetic_algorithm(NUM_GENERATIONS, POP_SIZE, NUM_PARAMS)

print("最优前三关节调控参数:", best_individual)

print("最小距离偏差（mm）:",RMSE(best_individual) )

print('最小克服重力势能做功（J）：',delta_Ep(best_individual))

'''求解后三个调控参数'''

#输出目标点在坐标系四中的坐标,为了求解关节4,5,6转角

def spacechange(x):

a1=(320*x[0]-160)*pi/180

a2=(165*x[1]-60)*pi/180

a3=(280*x[2]-200)*pi/180

l1=300

R2_1=np.array([[cos(a1),0,sin(a1),l1*cos(a1)],

[sin(a1),0,-cos(a1),l1*sin(a1)],

[0,1,0,600],

[0,0,0,1]])

l2=1200

R3_2=np.array([[cos(a2),-sin(a2),0,l2*cos(a2+pi/2)],

[sin(a2),cos(a2),0,l2*sin(a2+pi/2)],

[0,0,1,0],

[0,0,0,1]])

l3_4=1500

R4_3=np.array([[0,cos(a3),cos(pi/2+a3),l3_4*cos(pi/2+a3)],

[0,sin(a3),sin(pi/2+a3),sin(pi/2+a3)*l3_4],

[1,0,0,0],

[0,0,0,1]])

X=np.array([1500,1200,200,1])

R=R2_1@R3_2@R4_3

# 计算A的逆矩阵

R_inv = np.linalg.inv(R)

y=np.dot(R_inv, X)

return y

#输出后三位参数优化调控方向,输入前三个参数角度值

def lastthere(x):

P=spacechange(x)

x0=P[0]

y0=P[1]

z0=P[2]

cos=x0/sqrt(x0**2+y0**2)

a=acos(cos)

tan=z0/sqrt(x0**2+y0**2)

b=atan(tan)

a4=a-2*pi

a5=0

a6=pi/2-b

l=[a4,a5,a6]

return l

print('后三个角精细朝向调控参数为：',lastthere(best_individual))

'''求解能耗'''

import scipy.integrate as spi

'''求解能耗

theta0=[0,-pi/2,0,pi,-pi/2,0]

w_mean=[2.0,1.5,1.0,2.5,3.0,2.0]

I=[0.5,0.3,0.4,0.6,0.2,0.4]

def e_cost(x):

l=[]

for i in range(0,len(x)):

theta_f=x[i]+theta0[i]

w0=w_mean[i]

I0=I[i]

t=abs(x[i])*180/(pi*w0)

xita=np.array([theta0[i],theta_f,0,0])

Rt=np.array([[1,0,0,0],

[1,t,t**2,t**3],

[0,1,0,0],

[0,1,2*t,3*t**2]])

Rt_inv= np.linalg.inv(Rt)

aa=np.dot(Rt_inv,xita)

def omega_2(t):

return (aa[1]+2*aa[2]*t+3*aa[3]*t**2)**2

time=np.linspace(0,t,1000)

# 使用 quad 函数进行数值积分

result, error = spi.quad(omega_2, 0, t)  # 积分函数，积分下限，积分上限

r=result*0.5*I0/t

l.append(r)

return l

print('六个关节能耗为：',e_cost(theta))

'''

theta0 = [0, -pi/2, 0, pi, -pi/2, 0]

w_mean = [2.0, 1.5, 1.0, 2.5, 3.0, 2.0]

I = [0.5, 0.3, 0.4, 0.6, 0.2, 0.4]

def e_cost(theta):

l = []

for i in range(len(theta)):

if i!=4:

theta_f = theta[i] + theta0[i]

w0 = w_mean[i]

I0 = I[i]

t = abs(theta[i]) * 180 / (pi * w0)

xita = np.array([theta0[i], theta_f, 0, 0])

Rt = np.array([[1, 0, 0, 0],

[1, t, t**2, t**3],

[0, 1, 0, 0],

[0, 1, 2*t, 3*t**2]])

# 使用 numpy.linalg.solve 求解线性方程组，代替求逆

aa = np.linalg.solve(Rt, xita)

def omega_2(t):

return (aa[1] + 2 * aa[2] * t + 3 * aa[3] * t**2) ** 2

# 调整积分函数的定义域和精度

time = np.linspace(0, t, 1000)

result, error = spi.quad(omega_2, 0, t, epsabs=1.0e-6, epsrel=1.0e-6)  # 增加积分精度

r = result * 0.5 * I0 / t

l.append(r)

else:

l.append(0)

return l

t=lastthere(best_individual)

tt=np.array(best_individual)

theta=np.append(tt, t)

print('六个关节能耗为：', e_cost(theta))

第四问哈密尔顿圆代码

import heapq

class HamiltonianCycleSolver:

def __init__(self, graph):

self.graph = graph

self.num_vertices = len(graph)

self.min_cost = float('inf')

self.min_path = []

def solve(self):

# 使用优先队列（最小堆）来实现 Branch and Bound 算法

priority_queue = []

initial_path = [0]  # 从顶点0开始

initial_cost = 0

heapq.heappush(priority_queue, (initial_cost, initial_path))

while priority_queue:

current_cost, current_path = heapq.heappop(priority_queue)

# 如果当前路径已经包含了所有顶点，并且形成了一个哈密尔顿圆路径

if len(current_path) == self.num_vertices and self.graph[current_path[-1]][current_path[0]] != 0:

current_cost += self.graph[current_path[-1]][current_path[0]]  # 加上回到起点的权重

if current_cost < self.min_cost:

self.min_cost = current_cost

self.min_path = current_path[:]

continue

# 检查每个顶点是否可以加入到当前路径中

for vertex in range(self.num_vertices):

if (len(current_path) < self.num_vertices and

vertex not in current_path and

self.graph[current_path[-1]][vertex] != 0):

# 计算新路径的代价

new_cost = current_cost + self.graph[current_path[-1]][vertex]

# 只将代价小于当前最小代价的路径加入优先队列

if new_cost < self.min_cost:

new_path = current_path + [vertex]

heapq.heappush(priority_queue, (new_cost, new_path))

return self.min_cost, self.min_path

# 示例用法

if __name__ == "__main__":

# 示例带权图的邻接矩阵

graph = [

[0, 16, 17, 25,22,1000],

[16, 0, 11, 1000,1000,24],

[17, 11, 0, 10,1000,21],

[25, 1000, 10, 0,13,15],

[22,1000,1000,13,0,1000],

[1000,24,21,15,1000,0]

]

solver = HamiltonianCycleSolver(graph)

min_cost, min_path = solver.solve()

print(f"最小哈密尔顿圆路径的代价为: {min_cost}")

print(f"最小哈密尔顿圆路径为: {min_path}")